Detection and measurement logic

Inputs

- Time vector t and sweep signal y

- User-defined M window (m_start_ms to m_stop_ms)

- A time limit that prevents leakage into later responses (typically the start of the H window, h_start_ms)

Steps per sweep

- Windowing
Restrict analysis to the M window.

- Local extrema detection
Find indices of local maxima and minima using sign changes in the first derivative.

- Select the first extremum (P1 or N1)
Choose the earliest extremum in time within the M window.
This becomes the “primary” peak of the M-wave.

- Search for the opposite extremum
Search only after the primary peak and only within a constrained region:

  - after the first peak

  - before a cutoff time (typically h_start_ms, i.e., before H-reflex onset)
This step yields the “counter-peak” (N1 if the first was P1, or P1 if the first was N1).

- Fallback behavior
If no clean opposite local extremum exists (e.g., very shallow deflection), fall back to the global minimum/maximum within the constrained region.

- Amplitude computation
M_amp = abs(y[first_peak] - y[opposite_peak])

Output

- One M amplitude value per sweep, comparable across sweeps and stimulation levels.

Design decisions (and why)

1) Optional, not mandatory

Fixed-window peak-to-peak remains the default so results stay consistent with prior datasets and established workflows. The automatic mode can be enabled when waveform morphology requires it.

2) Measure a physiological biphasic pair, not the full window

The goal is the initial M-wave deflection (P1→N1). Measuring “first peak then first opposite peak” is more stable when later oscillations occur.

3) Strict time constraints to avoid overlap

The opposite-peak search is limited to prevent capturing:

- late oscillations within the M window, and

- early parts of other responses (especially anything near H latency).
Using h_start_ms as an upper bound is a simple and robust guardrail.

4) Deterministic behavior

The method avoids random components and uses simple derivative-based extrema detection so that identical input produces identical output.

5) Conservative fallback instead of forced detection

When the opposite peak is not clearly present, returning a fallback estimate (or NaN if insufficient data) is preferred over selecting a distant unrelated extremum.